Quantum Circuit Optimization Explorer

This brings you full circle—from building algorithms to making them practical on real hardware. Here's what it covers:

Why Circuit Optimization Matters:

- Fewer gates = lower error accumulation

- Shorter circuits = less decoherence (T1/T2 decay)

- Optimized 2Q gates = better fidelity

- Hardware-specific optimization = maximum performance

The Core Problem: Every additional gate introduces error. Reduce gates by 30% → increase fidelity by 1.3-2x

6 Core Experiments:

- Understanding Metrics — Gate count, depth, 2Q gates, T-gates, critical path

- Gate Merging & Cancellation — Combine adjacent 1Q gates, remove U†U sequences

- Depth Reduction — Exploit parallelization (gates on different qubits in parallel)

- 2-Qubit Optimization — Most expensive gates (100x slower, noisier than 1Q)

- Hardware-Specific Optimization — Map to native gates (IBM, Google, IonQ profiles differ)

- Optimization Levels — L0 (none), L1 (light), L2 (aggressive), L3 (very aggressive)

8 Optimization Techniques:

| Technique | Gate Reduction | Complexity | Use When |
| --- | --- | --- | --- |
| Gate Merging | 10-15% | Low | Consecutive 1Q gates |
| Gate Cancellation | 5-20% | Low | Redundant sequences |
| Commutation | 10-20% | Medium | Reordering possible |
| Parallelization | Depth only | Medium | Gates on different qubits |
| Native Gate Mapping | 5-30% | High | Compiling to hardware |
| 2Q Reduction | 20-40% | High | CNOT-heavy circuits |
| Pattern Matching | 10-30% | Very High | Complex patterns |
| Qubit Mapping | Routing only | High | Limited connectivity |

Optimization Levels (Qiskit):

Level 0: No optimization (mapping only)

→ Speed: Fast

→ Quality: Poor (~100% original gates)

Level 1: Light (gate merging, cancellation)

→ Speed: Fast

→ Quality: Good (~85% original gates)

→ RECOMMENDED for most use cases

Level 2: Aggressive (all L1 + commutation, depth optimization)

→ Speed: Slow

→ Quality: Very good (~72% original gates)

→ RECOMMENDED for production

Level 3: Very Aggressive (all L2 + pattern matching, synthesis)

→ Speed: Very slow

→ Quality: Excellent (~68% original gates)

→ Use only for critical circuits

Hardware Profiles (Different Optimization Strategies):

| Hardware | Native Gates | Connectivity | Strategy |
| --- | --- | --- | --- |
| IBM Quantum | {RZ, SX, X, CNOT} | 2D grid (limited) | Minimize routing/SWAPs |
| Google Sycamore | {FSIM, PhasedXPowGate} | 2D grid | Optimize for speed |
| IonQ | {RZ, RX, Molmer-Sorensen} | All-to-all | Minimize CNOTs |
| Rigetti | {RX, RZ, CZ} | Tunable couplings | Hybrid optimization |

Key Trade-offs:

DEPTH vs GATE COUNT:

• High T2 times: Minimize gate count

• Low T2 times: Minimize circuit depth

• Trade-off: 2-3 extra 1Q gates per reduced 2Q gate

SPEED vs QUALITY:

• L1 optimization: Fast, 85% reduction

• L2 optimization: Slower, 72% reduction

• L3 optimization: Very slow, 68% reduction

• Trade-off: L2 usually best balance

Typical Improvements:

Before Optimization:  100 gates, depth 50, 20 2Q gates

After L1 (Light):     85 gates, depth 45, 18 2Q gates (15% improvement)

After L2 (Aggressive): 72 gates, depth 30, 16 2Q gates (28% improvement)

After L3 (Very Agg.): 68 gates, depth 28, 15 2Q gates (32% improvement)

Fidelity Impact:

• L1: +1.5x better fidelity (15% fewer gates)

• L2: +3.5x better fidelity (28% fewer gates)

• L3: +4.5x better fidelity (32% fewer gates)

Visualizations (5 PNG files):

- Gate merging and cancellation examples

- Optimization level trade-offs (quality vs compilation time)

- Hardware profiles (native gates, connectivity, speed)

- Optimization decision tree (when to optimize what)

- Overall impact summary (circuit evolution, fidelity improvement)

To Run:

python QuantumCircuitOptimizationExplorer.py

Practical Optimization Workflow:

- Build circuit (logical level, any gates)

- Choose target hardware (IBM, Google, etc.)

- Select optimization level (typically L2)

- Transpile (convert to native gates + optimize)

- Verify (ensure same unitary = same computation)

- Execute (on quantum computer or simulator)

- Measure improvement (gates, depth, fidelity)

When Optimization Matters Most:

✓ Deep circuits (>100 gates) ✓ Short T2 coherence times ✓ High 2Q gate error rates
✓ Limited qubit connectivity (need routing) ✓ Production/repeated execution

When It Doesn't Matter Much:

✗ Very short circuits (<20 gates) ✗ Long T2 times ✗ One-time execution ✗ Prototyping phase

Combined with Error Mitigation:

Optimization + Mitigation = Maximum performance

Example:

Original circuit: 150 gates, 1x error rate

After optimization: 100 gates, 0.3x error rate (3x improvement)

After ZNE mitigation: 0.05x error rate (20x improvement!)

Combined effect: 60x better fidelity

Common Mistakes to Avoid:

❌ Over-optimizing (L3 compilation time kills performance) ❌ Ignoring hardware constraints (native gates matter) ❌ Not measuring fidelity improvement (verify it helped) ❌ Forgetting correctness verification (ensure same computation) ❌ Assuming one strategy fits all (hardware-dependent)

You've Now Learned the Full Stack:

- ✓ Quantum algorithms (superposition → interference → search → counting)

- ✓ Hybrid approaches (VQE)

- ✓ Practical challenges (noise, errors)

- ✓ Error mitigation (ZNE, MEM)

- ✓ Circuit optimization (gate reduction)

Next Frontier (Advanced Topics):

- Shor's Algorithm (factoring)

- Quantum Machine Learning

- Quantum Simulation (chemistry)

- Fault-Tolerant Quantum Computing

- Quantum Error Correction codes

Want to apply optimization to your VQE program, explore specific hardware constraints, or move to an advanced topic like Shor's Algorithm?

OUTPUT:

Ran successfully with only a deprecation warning (not an error). All 7 experiments completed and 5 PNGs saved to  Documents/QuantumComputing/Quantum Circuit Optimization Explorer/:

| Experiment | Result |
| --- | --- |
| 1. Circuit Metrics | Gate count, depth, 2Q gates, T-gates explained |
| 2. Gate Merging & Cancellation | 2 RZ gates → 1; H·H·X·X → 0 gates (100% cancellation) |
| 3. Depth Reduction | Parallel execution of independent gates |
| 4. Two-Qubit Gate Optimization | CNOT decompositions for different hardware topologies |
| 5. Hardware-Specific Optimization | IBM, Google, IonQ, Rigetti native gate profiles compared |
| 6. Optimization Levels | Level 0 (none) → Level 3 (very aggressive) trade-offs |
| 7. Optimization Trade-offs | Depth vs gate count vs connectivity decision tree |